- Charging @0.25C

Step 1: Understand the Parameters

- Battery capacity (Q) = 50 Ah

- Charge rate (C-rate) = 0.25 C

  - By definition, 1C means charging/discharging the full capacity in 1 hour.

  - So, 0.25C = charging current equal to 25% of the rated capacity per hour.

️ Step 2: Calculate Charging Current

I = C-rate* Q = 0.25 * 50 Ah = 12.5 ,

So, the charger is supplying 12.5 A continuously.

Step 3: Ideal Charging Time

In an ideal idle situation (no losses, 100% efficiency, constant current charging from 0% to 100%):

[ t = Q/I = {50}/{12.5} = 4 hours]

So, theoretically, it will take 4 hours to fully charge.

️ Step 4: Real-World EV Designer Considerations

In practice, charging is not linear:

- CC-CV profile: Lithium-ion batteries are charged in two stages:

  - Constant Current (CC) until ~80–90% SOC.

  - Constant Voltage (CV) where current tapers off, extending the last 10–20% of charging.

- Efficiency losses: Heat, conversion losses, and balancing circuits add ~5–15% overhead.

- Idle situation assumption: Since you specified “idle situation,” we ignore load and thermal effects, so we stick to the ideal 4 hours.

- Ideal case (idle, 100% efficient, CC only): 4 hours.

- Real EV design case (CC-CV, efficiency losses): closer to 4.5–5 hours to reach 100% SOC.

* In EV design, we rarely allow charging from 0% to 100%. Typically, SOC window is 10–90% to extend battery life. That means in practice, your usable charging time would be ~3.2 hours at 0.25C.

What is your laptop battery cycle count and how many times your laptop is charged?

## Step 1: Key Concepts

- Cycle count (N): Number of full charge–discharge cycles completed.

- Depth of discharge (DoD): How much of the battery capacity is used per cycle (e.g., 100% DoD = full discharge, 50% DoD = half discharge).

- Cycle life rating (Nₘₐₓ): Manufacturer’s rated cycles until capacity drops to ~80%.

## ️ Step 2: General Formula

- A simplified model for remaining capacity:

- Cremaining = C rated* (1- N/N_max*f(DoD))Where:

- (C_{rated}) = original capacity (e.g., 50 Ah for EV, 50 Wh for laptop).

- (N) = actual cycle count.

- (N_max) = rated cycle life (e.g., 500 cycles for laptop, 3000 cycles for EV).

- (f(DoD)) = correction factor for partial discharges.

## Step 3: DoD Correction Factor

- Full discharge (100% DoD): (f(DoD) = 1).

- Partial discharge (e.g., 50% DoD): (f(DoD) \approx 0.5).

- Rule of thumb: Shallower cycles extend life.

  - Example: 1000 cycles at 100% DoD ≈ 2000 cycles at 50% DoD.

## Step 4: Example Calculation

- Laptop battery:

- Rated cycle life: 500 cycles.

- Current cycle count: 250 cycles.

- DoD: ~80% (typical laptop use).

- [ C_{remaining} = 100% \times \left(1 - \frac{250}{500} \cdot 0.8\right) = 60% ]

- So, the battery has ~60% of its original capacity left.

- EV battery pack:

- Rated cycle life: 3000 cycles.

- Current cycle count: 1000 cycles.

- DoD: 80% (common EV window).

- [ C_{remaining} = 100% \times \left(1 - \frac{1000}{3000} \cdot 0.8\right) \approx 73% ]

- So, the EV pack retains ~73% capacity.

## Professional Insight

- Laptop batteries: degrade faster due to higher DoD and thermal stress.

- EV packs: managed by BMS (Battery Management System) to limit DoD (10–90%), extending cycle life.

- Rule of thumb: Every 300–500 full cycles → ~20% capacity loss in laptops. EVs stretch this to thousands of cycles.

- To open the battery status report powercfg/batteryreport

- The status will be shown as below:

Additionally it also shows how much power is drained:

- Here Energy Drained will be used to calculate Energy Remaining

- E.g. Cremaining = 100% (1-(0,20450/1)*0.25) =

- cycle count estimation from energy values (mWh) is a common task in both laptop battery diagnostics and EV battery pack analysis.

- Step 1: Understand the Definitions

- Rated capacity (Eᵣ): The manufacturer’s specified energy capacity of the battery, usually in mWh (e.g., 50,000 mWh).

- Drained capacity (E_d): The actual energy discharged during usage, also in mWh.

- Cycle count (N): Number of equivalent full cycles the battery has undergone.

- ️ Step 2: Formula for Equivalent Cycles

- A cycle is defined as one full discharge equal to the rated capacity.

- So, if you drain less than full capacity, you accumulate partial cycles.

- N={E_d}/{E_r}

- Where:

- E_d = total drained capacity over time (sum of all discharges).

- E_r = rated capacity of the battery.

- Step 3: Example Calculation

- Suppose:

- Rated capacity = 50,000 mWh

- Drained capacity (over several uses) = 25,000 mWh

- N={25,000}/{50,000}=0.5, mathrm{cycles}

- That means you’ve used half a cycle.

- If later you drain another 25,000 mWh, the total becomes:

- N={50,000}/{50,000}=1\{cycle}

- So two half discharges = one full cycle.

- ️ Step 4: Real-World EV/Laptop Considerations

- Partial discharges accumulate → 4 × 25% discharges = 1 cycle.

- Depth of discharge (DoD) matters → shallow cycles extend life.

- Battery management systems (BMS) log cycles using this equivalent full cycle method.

- Expert-Level Insight

- In EV design, we often track Energy Throughput (ET):

- ET=sum E_d

  - Then divide by rated capacity to get cycle count.

  - This method is more accurate than simply counting “plug-ins,” because it accounts for partial charges/discharges.

- E.g - N = 1316/6435 = 0.20450660 Cycles